Universitat Politècnica de Catalunya

Escola Superior d’Enginyeria Industrial, Aeroespacial i Audiovisual de Terrassa

Màster Universitari en Enginyeria Aeronàutica

The Chocolate Factory

Students:; Pol Fontanes Molina. Víctor Martínez Viol.

Course:; Applied Robotics. Lab. Group:; Group 5. Professors:; Jaume Figueras Jove.

Submission Date:; 3rd April 2019.

Table of Contents

1	Objectives	3

2	Laboratory Setup	3

3	Station Description	3

4	Program Documentation	3

4.1	Program Execution Strategy	4

4.1.1	How the Priority is Handled?	4

4.2	Flow Diagrams	5

4.3	Code Description	6

4.3.1	Interruptions	7

4.3.2	Functions and Procedures	8

4.3.3	Variables	10

5	Conclusions	10

6	Bibliography	10

Table of Figures

Figure 1: Laboratory environment components. IRB140 robot arm and IRC5 controller at the left and the right respectivelly.	3

Figure 2. Main program flow diagram	5

Figure 3. Perform task switch diagram	6

Figure 4. Tasks queue diagram	6

Figure 5: Teach Pendant.	8

Table of Tables

Table 1: Sample’s geometrical data and test conditions	4

Table 2. Global variables	10

# Objectives

# Laboratory Setup

In the Robotics and CIM laboratory (ESAII department, building TR11) there is an environment conformed by an ABB IRB140 robot arm, the robot controller (IRC5), the teach pendant and a pc connected with the robot controller by means of an Ethernet connection.

Figure 1: Laboratory environment components. IRB140 robot arm and IRC5 controller at the left and the right respectivelly.

# Station Description

The laboratory practice consists on a drying station compound of three main positions:

## Drying Station

Consisting of a drying oven which has 9 slots for drying the chocolate moulds in a configuration of 3 by 3.

## Conveyors

Integrated by two conveyors, one from where the chocolate moulds arrive and the other where the emptied chocolate moulds are returned.

## Manipulation Area

A manipulation stations where chocolates are removed from its mould.

# Program Documentation

The following section aim is to explain not only the variables defined and the syntax of the implemented functions but also the program execution flow and how are solved the different issues that may arise when programming a robot arm that work with asynchronous inputs.

The first part of the section covers the strategy applied in order to deal with different robot tasks / movements. Then, the execution flow diagrams of the programs are shown to provide a quick understanding about how the program is structured. Finally, a brief description of the code is done.

## Program Execution Strategy

As it has been shown in section 2, there are several tasks that must be done following a sequential order. Besides, during any step of this sequence, a new chocolate mould can arrive to the input conveyor and thus altering the task sequence.

The methodology followed in this work to deal with a new task appearing while the previous sequence is not yet finished is to define a queue where the different tasks are listed and they are executed following a criterion based on two factors: (a) Time and (b) Priority.

To do so, a unique ID has been assigned to each task and their completion times and priorities (from 0 to 10) have been defined. The following table lists the aforementioned values.

| Task | ID | Completion Time (s) | Priority |
| --- | --- | --- | --- |
| Pick a chocolate mould from the input conveyor and bring it to the drying warehouse. | 1 | 0 | 0 |
| Take the chocolate mould from the drying warehouse and bring it to the manipulation station. | 2 | 60 * 120 ** | 6 |
| Take the mould from the manipulation station and bring it to the output conveyor | 3 | 5 | 10 |
| No task to do | 0 | 0 | 0 |
| *if chocolate type is 1, **if chocolate type is 2 |  |  |  |

Table 1: Sample’s geometrical data and test conditions

All this information has to be available for the program to stablish an order of execution for the different tasks. Besides, as there are some tasks that take some time to be completed, the robot must know exactly the time at when the tasks are finished. This information is contained inside a queue matrix which is called taskQueue. This matrix stores the task ordered by descending completion time (considering also the priority) and has the following columns:

- Task Id: To Correctly identify the task.

- Completion time: Stored in seconds since 00:00.

- Optional parameter 1: Optional parameter, used to distinguish between chocolate type.

- Optional parameter 2: Optional parameter, used to identify the position of the mould in the drying warehouse.

- Priority: Priority integer, used when ordering the tasks.

### How the Priority is Handled?

As it will be further explained in section 4.2., the code developed uses a for-loop to insert a new task inside the queue. It compares the completion time of the new task with the completion time of the tasks already in the queue. The strategy applied to also consider the priority in this comparison is to convert the priority into an artificial time delay to be added to the completion time of the tasks. Therefore, a task with the maximum priority will use its real completion time whereas a task with less priority will use its completion time plus an artificial increment of time when comparing it. This will guarantee that even though the latter is completed before, it will be situated after the task with higher priority in the queue.

## Flow Diagrams

Figure 2. Main program flow diagram

Figure 3. Perform task switch diagram

Figure 4. Tasks queue diagram

## Code Description

This section is intended to contain a brief description of the functions and procedures implemented in the code.

### Interruptions

Three interruptions have been developed in this code. One (iStop) is used to deal with an emergency stop and the others (iMove1, iMove2) are used to tell the robot that a chocolate mould has arrived.

Below these lines, two code snippets can be found. The first contains the definition of the interrupt variables and their link with the interruption methods. The second, show the implementation of the interruptions aforementioned.

| 62 63 64 65 79 96 97 98 99 100 101 102 130 | !Interrupts VAR intnum pushInt1; VAR intnum pushInt2; VAR intnum pushInt3; ! […] PROC main() ! […] ! 2. Connect interrupts CONNECT pushInt1 WITH iMove1; ISignalDI sensor1,1,pushInt1; CONNECT pushInt2 WITH iMove2; ISignalDI sensor2,1,pushInt2; CONNECT pushInt3 WITH iStop; ISignalDI sensor3,1,pushInt3; ! […] ENDPROC |
| --- | --- |

| 138 139 140 141 142 143 144 145 146 147 148 | TRAP iMove1 triggerSeq2 1, taskQueue, taskTimming; ENDTRAP TRAP iMove2 triggerSeq2 2, taskQueue, taskTimming; ENDTRAP TRAP iStop emergencyStop; ENDTRAP |
| --- | --- |

The interruptions are connected to the digital signals sensor1, sensor2 and sensor3. These signals have been linked to the robot teach pendant programmable keys 1 to 3 shown in the Figure 2.

Figure 5: Teach Pendant.

### Functions and Procedures

#### checkPos

| Syntax: | checkPos() |
| --- | --- |
| Description: | Reads the current position of the arm (using the built-in function CPos() and computes the difference between this position and the reference position of the four key positions of the station. It returns a num code if the difference is smaller than a given threshold. Return: 0: Home; 1: Conveyor; 2: Drying warehouse; 3: manipulation station; 404: The position does not correspond any of the above. |

#### defineOvenPoints

| Syntax: | defineOvenPts(orig, [x, y, z], [xx, yy, zz], pts) |
| --- | --- |
| Description: | Construct the drying warehouse matrix by taking a reference point called orig, an offset between warehouse positions [x, y, z] and a security offset between the security points and the warehouse points [xx, yy, zz]. This matrix is then stored in a 2x3x3 matrix called pts. |

#### defineConvPoints

| Syntax: | defineConvPts(orig, [x, y, z], [xx, yy, zz], pts) |
| --- | --- |
| Description: | Construct the input and output conveyors position by taking a reference point called orig, an offset between the two conveyors [x, y, z] and a security offset between the security points and the conveyors [xx, yy, zz]. This matrix is then stored in a 2x2 matrix called pts. |

#### conv2oven

| Syntax: | conv2oven(p, q, k, i, j) |
| --- | --- |
| Description: | This function is responsible for the movement between the conveyor number k at position p (2x2 matrix) and the position i,j of the drying warehouse whose points are stored in q (2x3x3 matrix). |

#### oven2man

| Syntax: | Oven2man(p, q, i, j) |
| --- | --- |
| Description: | This function is responsible for the movement between the position i,j of the drying warehouse whose points are stored in q (2x3x3 matrix) and the manipulation station q (2x1 matrix). |

#### man2conv

| Syntax: | man2conv(p, q, i) |
| --- | --- |
| Description: | This function is responsible for the movement between the manipulation station p (2x1 matrix) and the conveyor number i at position q (2x2 matrix). |

#### triggerSeq2

| Syntax: | triggerSeq2(type, queue, time) |
| --- | --- |
| Description: | Creates a task of type type and adds it to the queue of tasks queue. It also triggers updateDisp |

#### updateDisp

| Syntax: | updateDisp(n) |
| --- | --- |
| Description: | Clears the teach pendant display and prints the chocolate counters identified by the matrix n “CHOCOLATE TYPE 1: Produced = x Total = x CHOCOLATE TYPE 2: Produced = y Total = Y” |

#### emergencyStop

| Syntax: | emergencyStop() |
| --- | --- |
| Description: | Sends a break message to the program thread and prints a message in the teach pendant display. |

#### performTask

| Syntax: | performTask(queue, occOven, time, n) |
| --- | --- |
| Description: | This function executes the first task on the task queue queue, creates a new task and updates the queue. It uses the drying warehouse occupancy stored at occOven, the task timings time and the chocolate counters n. |

#### moveS

| Syntax: | moveS(p, vFree, vSec, z, t) |
| --- | --- |
| Description: | For a pair of points (security point and actual point) defined as p, this function performs the following movements with zonedata z and tooldata t: - MoveJ to the security point with velocity vFree. - MoveJ to the actual point with velocity vSec. - MoveJ to the security point with velocity vSec. |

#### movTest

| Syntax: | movTest(p, q, r) |
| --- | --- |
| Description: | This function executes different movements sequences between the Home position and the positions stored at the matrices p, q and r. |

#### gotoOvenPts

| Syntax: | gotoOvenPts(p) |
| --- | --- |
| Description: | Move the robot sequentially between the positions defined in the matrix p. |

### Variables

There are different global variables used on the program, listed below:

| Type | Variable | Function |
| --- | --- | --- |
| robtarget | pHome | Home reference coordinates |
| robtarget | pConvRef | First conveyor reference coordinates |
| robtarget | pManRef | Manipulation station reference coordinates |
| robtarget | pOvenRef | First oven slot reference coordinates |
| num | convSecOffset{3} | Conveyor security offset |
| num | manSecOffset{3} | Manipulation station security offset |
| num | ovenSecOffset{3} | First oven slot security offset |
| num | convOffset{3} | Offset between the two conveyors |
| num | ovenMatOffset{3} | Offset between oven positions |
| speeddata | vSecurity | Security movement velocity |
| num | taskTimming{4} | Time in seconds to trigger next task |
| num | timeDelta | Time difference between currTime and next Task |
| num | timeMov | Time elapsed during robot movements |
| num | currTime | Current execution time |
| bool | occOven{9} | Oven slots ids computed as (i-1) *3+1 |
| bool | isHome | Flag variable to know if arm is at home position |
| robtarget | pConv{2,2} | Vector of coordinates for the conveyor (offset and final position) |
| robtarget | pOven{2,3,3} | Vector of coordinates for the oven slots (offset and final position) |
| robtarget | pMan{2} | Vector of coord. for the manipulation area (offset and final position) |
| num | numChoc{2,2} | Vector of produced and to produce chocolate chips |
| num | chocType | Chocolate type identifier |
| intnum | pushInt1 | Interrupt variable 1 |
| intnum | pushInt2 | Interrupt variable 2 |
| intnum | pushInt3 | Interrupt variable 3 |
| pos | pos1 | Actual position variable |
| num | place | Place id |

Table 2. Global variables

# Conclusions

# Bibliography

Per si necessitem snippets del codi

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 117 118 119 120 121 122 123 124 125 126 127 128 129 130 131 132 133 134 135 136 137 138 139 140 141 142 143 144 145 146 147 148 149 150 151 152 153 154 155 156 157 158 159 160 161 162 163 164 165 166 167 168 169 170 171 172 173 174 175 176 177 178 179 180 181 182 183 184 185 186 187 188 189 190 191 192 193 194 195 196 197 198 199 200 201 202 203 204 205 206 207 208 209 210 211 212 213 214 215 216 217 218 219 220 221 222 223 224 225 226 227 228 229 230 231 232 233 234 235 236 237 238 239 240 241 242 243 244 245 246 247 248 249 250 251 252 253 254 255 256 257 258 259 260 261 262 263 264 265 266 267 268 269 270 271 272 273 274 275 276 277 278 279 280 281 282 283 284 285 286 287 288 289 290 291 292 293 294 295 296 297 298 299 300 301 302 303 304 305 306 307 308 309 310 311 312 313 314 315 316 317 318 319 320 321 322 323 324 325 326 327 328 329 330 331 332 333 334 335 336 337 338 339 340 341 342 343 344 345 346 347 348 349 350 351 352 353 354 355 356 357 358 359 360 361 362 363 364 365 366 367 368 369 370 371 372 373 374 375 376 377 378 379 380 381 382 383 384 385 386 387 388 389 390 391 392 393 394 395 396 397 398 399 400 401 402 403 404 405 406 407 408 409 410 411 412 413 414 415 416 417 418 419 420 421 422 423 424 425 426 427 428 429 430 431 432 433 434 435 436 437 438 439 440 441 442 443 444 445 446 447 448 449 450 451 452 453 454 455 456 457 458 459 460 461 462 463 464 465 466 467 468 469 470 471 472 473 474 475 476 477 478 479 480 481 482 483 484 485 486 487 488 489 490 491 492 493 494 495 496 497 498 499 500 501 502 503 504 505 506 507 508 509 510 511 512 513 514 515 516 517 518 519 520 521 522 523 | MODULE ObelixMov !*********************************************************** ! ! Module: ObelixMov ! ! Description: ! Move the robot ! ! Author: pol & victor ! ! Version: 0.1.0 ! !*********************************************************** CONST robtarget pHome :=[[507.9,-6.43,715.02],[0.697685,-0.00154316,0.716328,-0.0103435],[-1,-1,-1,0],[9E+09,9E+09,9E+09,9E+09,9E+09,9E+09]]; CONST robtarget pConvRef :=[[-109.516885301,-504.792720334,376.620999376],[0.243054291,0.717735721,0.578717345,-0.301440343],[-2,0,-1,0],[9E+09,9E+09,9E+09,9E+09,9E+09,9E+09]]; CONST robtarget pManRef :=[[607.9,-6.43,695.02],[0.697685,-0.00154316,0.716328,-0.0103435],[-1,-1,-1,0],[9E+09,9E+09,9E+09,9E+09,9E+09,9E+09]]; !CONST robtarget pManRef :=[[507.9,-6.43,715.02],[0.697685,-0.00154316,0.716328,-0.0103435],[-1,-1,-1,0],[9E+09,9E+09,9E+09,9E+09,9E+09,9E+09]]; CONST robtarget pOvenRef :=[[-291.029880742,659.908842444,594.297264732],[0.363486279,-0.625201176,0.407577419,0.55756781],[1,0,-1,0],[9E+09,9E+09,9E+09,9E+09,9E+09,9E+09]]; CONST num convSecOffset{3} := [0, 0, -100]; !security offset [x, y, z] CONST num manSecOffset{3} := [50, 0, -50]; !security offset [x, y, z] CONST num ovenSecOffset{3} := [0, 100, 0]; !security offset [x, y, z] CONST num convOffset{3} := [0, -200, 0]; !offset between 2 conv [x, y, z] CONST num ovenMatOffset{3} := [100, 0, -100]; !offset between oven positions [x, y, z] CONST speeddata vSecurity := v1000; ! Task variables ! +---------+------------------------------------------------+ ! ¦ Task ID ¦ Task Description ¦ ! +---------+------------------------------------------------+ ! ¦ 1 ¦ pick choco from conveyor1 & bring to oven ¦ ! ¦ 2 ¦ take choco from oven & bring to man. station ¦ ! ¦ 3 ¦ take the mould from ms & throw it to conveyor2 ¦ ! ¦ 0 ¦ no task ¦ ! +---------+------------------------------------------------+ VAR num taskTimming{4} := [30, 30, 0, 5]; ! time in seconds to trigger next task VAR num taskPrior{4} := [0, 6, 10, 0]; VAR num timeDelta := 0; !delta between currTime and next Task VAR num priorDel := 20; !priority artificial time VAR num timeMov := 30; !time elapsed during robot movements VAR num taskQueue{30,5}; ![Task id, completion time, opt_par1(chocType), opt_par2, priority] VAR num currTime; ! var to store current time VAR bool occOven{9}; ! idx computed as (i-1)*3+j VAR bool isHome := TRUE; ! flag to know if the robot is at pHome ! Points variables VAR robtarget pConv{2,2}; VAR robtarget pOven{2,3,3}; VAR robtarget pMan{2}; ! Flexpendant vars VAR num numChoc{2,2}; !{1,*} num produced; {2,*} Total to produce VAR num chocType; !Interrupts VAR intnum pushInt1; VAR intnum pushInt2; VAR intnum pushInt3; !Check position VAR pos pos1; VAR num place; !*********************************************************** ! ! Procedure main ! ! This is the entry point of your program ! !*********************************************************** PROC main() MoveJ pHome, v1000, fine, tool0; !----------------------------------- ! 0. Points definition pMan := [pManRef, Offs(pManRef, manSecOffset{1}, manSecOffset{2}, manSecOffset{3})]; defineConvPts pConvRef, convOffset, convSecOffset, pConv; defineOvenPts pOvenRef, ovenMatOffset, ovenSecOffset, pOven; ! 1. Job Configuration TPErase; TPWrite "Welcome to the chocolate factory"; TPReadNum numChoc{2,1}, "How many chocolate 1 items will be produced?"; TPReadNum numChoc{2,2}, "How many chocolate 2 items will be produced?"; updateDisp numChoc; ! 2. Connect interrupts CONNECT pushInt1 WITH iMove1; ISignalDI sensor1,1,pushInt1; CONNECT pushInt2 WITH iMove2; ISignalDI sensor2,1,pushInt2; CONNECT pushInt3 WITH iStop; ISignalDI sensor3,1,pushInt3; ! 3. Reescale the task priority FOR i FROM 1 TO Dim(taskPrior,1) DO taskPrior{i} := (taskPrior{i} - 0) * (0 - 10) / (10 - 0) + 10; ENDFOR ! 4. Start the job !while produced < total WHILE numChoc{1,1}<numChoc{2,1} OR numChoc{1,2}<numChoc{2,2} DO !get current time currTime := GetTime(\Hour)*3600 + GetTime(\Min)*60 + GetTime(\Sec); !do some movement IF taskQueue{1,1} <> 0 AND taskQueue{1,2} - currTime < timeDelta THEN performTask taskQueue, occOven, taskTimming, numChoc, pConv, pOven, pMan; isHome := FALSE; ELSEIF (NOT isHome) AND (taskQueue{1,1} = 0 OR taskQueue{1,2} - currTime > timeMov) THEN MoveJ pHome, v1000, fine, tool0; isHome := TRUE; ENDIF ENDWHILE ! TEST. Movement Tests !movTest pConv, pOven, pMan; !----------------------------------- MoveJ pHome, v1000, fine, tool0; ENDPROC !*********************************************************** ! @deprecated TRAP iMove triggerSeq chocType, taskQueue, taskTimming; ENDTRAP TRAP iMove1 triggerSeq2 1, taskQueue, taskTimming; ENDTRAP TRAP iMove2 triggerSeq2 2, taskQueue, taskTimming; ENDTRAP TRAP iStop emergencyStop; ENDTRAP !*********************************************************** FUNC num checkPos() VAR pos check_diff; VAR num diff; VAR num threshold := 250; !0-> home 1->conveyor 2->oven 3->man !check home pos1 := CPos(); check_diff.x := pos1.x - pHome.trans.x; check_diff.y := pos1.y - pHome.trans.y; check_diff.z := pos1.z - pHome.trans.z; diff := VectMagn(check_diff); IF diff <= threshold THEN RETURN 0; ENDIF !check conveyor pos1 := CPos(); check_diff.x := pos1.x - pConvRef.trans.x; check_diff.y := pos1.y - pConvRef.trans.y; check_diff.z := pos1.z - pConvRef.trans.z; diff := VectMagn(check_diff); IF diff <= threshold THEN RETURN 1; ENDIF !check oven pos1 := CPos(); check_diff.x := pos1.x - pOvenRef.trans.x; check_diff.y := pos1.y - pOvenRef.trans.y; check_diff.z := pos1.z - pOvenRef.trans.z; diff := VectMagn(check_diff); IF diff <= threshold THEN RETURN 2; ENDIF !check man pos1 := CPos(); check_diff.x := pos1.x - pManRef.trans.x; check_diff.y := pos1.y - pManRef.trans.y; check_diff.z := pos1.z - pManRef.trans.z; diff := VectMagn(check_diff); IF diff <= threshold THEN RETURN 3; ENDIF !DEFAULT RETURN 404; ENDFUNC !*********************************************************** PROC defineOvenPts(robtarget orig, num matOffs{*}, num secOffs{*}, INOUT robtarget pts{*,*,*}) !define the security points {1} and the inside oven points {2} FOR i FROM 1 TO 3 DO FOR j FROM 1 TO 3 DO pts{1,i,j} := Offs(orig, matOffs{1}*(i-1), matOffs{2}, matOffs{3}*(j-1)); pts{2,i,j} := Offs(pts{1,i,j}, secOffs{1}, secOffs{2}, secOffs{3}); ENDFOR ENDFOR ENDPROC !*********************************************************** PROC defineConvPts(robtarget orig, num convOffs{*}, num secOffs{*}, INOUT robtarget pts{*,*}) !define the security points {1} and the conv points {2} FOR i FROM 1 TO 2 DO pts{1,i} := Offs(orig, convOffs{1}*(i-1), convOffs{2}*(i-1), convOffs{3}*(i-1)); pts{2,i} := Offs(pts{1,i}, secOffs{1}, secOffs{2}, secOffs{3}); ENDFOR ENDPROC !*********************************************************** PROC gotoOvenPts(robtarget pts{*,*,*}) FOR i FROM 1 TO 3 DO FOR j FROM 1 TO 3 DO MoveS [pOven{1, i, j}, pOven{2, i, j}], v1000, v200, fine, tool0; ENDFOR ENDFOR ENDPROC !*********************************************************** PROC conv2oven(robtarget pConv{*,*}, robtarget pOven{*,*,*}, num iConv, num iOven, num jOven) !Check position place := checkPos(); IF place = 2 OR place = 404 THEN MoveJ pHome, v1000, fine, tool0; ENDIF MoveS [pConv{1, iConv}, pConv{2, iConv}], v1000, vSecurity, fine, tool0; MoveJ pHome, v1000, fine, tool0; MoveS [pOven{1, iOven, jOven}, pOven{2, iOven, jOven}], v1000, vSecurity, fine, tool0; ENDPROC !*********************************************************** PROC oven2man(robtarget pOven{*,*,*}, robtarget pMan{*}, num iOven, num jOven) !Check position place := checkPos(); IF place = 1 OR place = 404 THEN MoveJ pHome, v1000, fine, tool0; ENDIF MoveS [pOven{1, iOven, jOven}, pOven{2, iOven, jOven}], v1000, vSecurity, fine, tool0; MoveJ pHome, v1000, fine, tool0; MoveS pMan, v1000, vSecurity, fine, tool0; ENDPROC !*********************************************************** PROC man2conv(robtarget pMan{*}, robtarget pConv{*,*}, num iConv) MoveS pMan, v1000, vSecurity, fine, tool0; MoveJ pHome, v1000, fine, tool0; MoveS [pConv{1, iConv}, pConv{2, iConv}], v1000, vSecurity, fine, tool0; ENDPROC !*********************************************************** ! @deprecated PROC triggerSeq(INOUT num type, INOUT num queue{*,*}, num time{*}) VAR num newTask{5}; VAR num auxTask{5}; VAR num currTime; currTime := GetTime(\Hour)*3600 + GetTime(\Min)*60 + GetTime(\Sec); TPReadFK type, "A chocolate figure has arrived to the station. Which type of chocolate is?", "TP1", "TP2", stEmpty,stEmpty,stEmpty; !Add a task 1 to the queue currTime := GetTime(\Hour)*3600 + GetTime(\Min)*60 + GetTime(\Sec); newTask := [1, currTime, type, 0, taskPrior{1}]; FOR i FROM 1 TO Dim(queue, 1) DO IF newTask{2} + priorDel * newTask{5} + timeMov < queue{i,2} + priorDel * queue{i,5} OR queue{i,1} = 0 THEN !backup newTask auxTask := newTask; newTask{1} := queue{i,1}; newTask{2} := queue{i,2}; newTask{3} := queue{i,3}; newTask{4} := queue{i,4}; newTask{5} := queue{i,5}; queue{i,1} := auxTask{1}; queue{i,2} := auxTask{2}; queue{i,3} := auxTask{3}; queue{i,4} := auxTask{4}; queue{i,5} := auxTask{5}; ENDIF ENDFOR ENDPROC !*********************************************************** PROC triggerSeq2(num type, INOUT num queue{*,*}, num time{*}) VAR num newTask{5}; VAR num auxTask{5}; VAR num currTime; updateDisp numChoc; TPWrite "A chocolate figure TYPE" \Num:=type; TPWrite "has arrived to the station"; !Add a task 1 to the queue currTime := GetTime(\Hour)*3600 + GetTime(\Min)*60 + GetTime(\Sec); newTask := [1, currTime, type, 0, taskPrior{1}]; FOR i FROM 1 TO Dim(queue, 1) DO IF newTask{2} + priorDel * newTask{5} + timeMov < queue{i,2} + priorDel * queue{i,5} OR queue{i,1} = 0 THEN !backup newTask auxTask := newTask; newTask{1} := queue{i,1}; newTask{2} := queue{i,2}; newTask{3} := queue{i,3}; newTask{4} := queue{i,4}; newTask{5} := queue{i,5}; queue{i,1} := auxTask{1}; queue{i,2} := auxTask{2}; queue{i,3} := auxTask{3}; queue{i,4} := auxTask{4}; queue{i,5} := auxTask{5}; ENDIF ENDFOR ENDPROC !*********************************************************** PROC updateDisp(num n{*,*}) TPErase; TPWrite "CHOCOLATE TYPE 1:"; TPWrite " Produced = ", \Num := n{1,1}; TPWrite " Total = ", \Num := n{2,1}; TPWrite "CHOCOLATE TYPE 2:"; TPWrite " Produced = ", \Num := n{1,2}; TPWrite " Total = ", \Num := n{2,2}; !TPWrite " Position = ", \Pos := pos1; !TPWrite " Position according to us = ", \Num := place; ENDPROC !*********************************************************** PROC emergencyStop() TPWrite "EMERGENCY BUTTON PRESSED"; Break; ENDPROC !*********************************************************** PROC MoveS(robtarget p{*}, speeddata vFree, speeddata vSec, zonedata z, PERS tooldata t) MoveJ p{1}, vFree, z, t; MoveJ p{2}, vSec, z, t; MoveJ p{1}, vSec, z, t; ENDPROC !*********************************************************** PROC performTask(INOUT num queue{*,*}, INOUT bool occOven{*}, num time{*}, INOUT num n{*,*}, robtarget pConv{*,*}, robtarget pOven{*,*,*}, robtarget pMan{*}) VAR num iOven; VAR num jOven; VAR bool found := FALSE; VAR num newTask{5}; ![taskID, time, opt1, opt2, priority] VAR num currTime; !Switch TaskID TEST queue{1,1} CASE 1: ! CHOCO HAS ARRIVED !check the 1st empty position on the oven FOR i FROM 1 TO 9 DO IF (NOT occOven{i}) AND (NOT found) THEN found := TRUE; iOven := i DIV 3; jOven := i MOD 3; occOven{i} := TRUE; !hack: avoid 0s IF iOven = 0 iOven := 1; IF jOven = 0 jOven := 3; !Break; there's no break :( ENDIF ENDFOR conv2oven pConv, pOven, 1, iOven, jOven; !generate a new task currTime := GetTime(\Hour)*3600 + GetTime(\Min)*60 + GetTime(\Sec); newTask := [2, currTime+time{queue{1,1}+(queue{1,3}-1)}, queue{1,3}, 3*(iOven-1)+jOven, taskPrior{queue{1,1}}]; CASE 2: !PICK FROM OVEN & BRING TO MAN !get the oven position iOven := queue{1,4} DIV 3; jOven := queue{1,4} MOD 3; !hack: avoid 0s IF iOven = 0 iOven := 1; IF jOven = 0 jOven := 3; occOven{queue{1,4}} := FALSE; !perform the task oven2man pOven, pMan, iOven, jOven; !generate a new task currTime := GetTime(\Hour)*3600 + GetTime(\Min)*60 + GetTime(\Sec); newTask := [3, currTime+time{queue{1,1}+1}, queue{1,3}, 0, taskPrior{queue{1,1}}]; CASE 3: !TAKE MOULD AND BRING TO CONVEYOR !perform the task man2conv pMan, pConv, 2; !generate a new task newTask := [0, 0, 0, 0, taskPrior{queue{1,1}}]; !Update chocolate counters IF queue{1,3} = 1 THEN Incr n{1,1}; ELSE Incr n{1,2}; ENDIF !Erase the contents of the display and print the numbers of figures completed updateDisp n; DEFAULT: !do nothing and exit the proc RETURN; ENDTEST !update the queue list comparing the completion times FOR i FROM 2 TO Dim(queue, 1) DO IF newTask{1} = 0 OR (queue{i,1} <> 0 AND queue{i,2} + priorDel * queue{i,5} < newTask{2} + priorDel * newTask{5} + timeMov) THEN queue{i-1,1} := queue{i,1}; queue{i-1,2} := queue{i,2}; queue{i-1,3} := queue{i,3}; queue{i-1,4} := queue{i,4}; queue{i-1,5} := queue{i,5}; ELSE !newTask completes before the queued task queue{i-1,1} := newTask{1}; queue{i-1,2} := newTask{2}; queue{i-1,3} := newTask{3}; queue{i-1,4} := newTask{4}; queue{i-1,5} := newTask{5}; RETURN;!Break; There's no break :( ENDIF ENDFOR ENDPROC !*********************************************************** PROC movTest(robtarget pConv{*,*}, robtarget pOven{*,*,*}, robtarget pMan{*}) ! Perform diferent movement tests !oven2oven gotoOvenPts pOven; !conv2oven FOR i FROM 1 TO 3 DO FOR j FROM 1 TO 3 DO conv2oven pConv, pOven, 1, i, j; ENDFOR ENDFOR !oven2man FOR i FROM 1 TO 3 DO FOR j FROM 1 TO 3 DO oven2man pOven, pMan, i, j; ENDFOR ENDFOR !man2conv man2conv pMan, pConv, 2; !oven2home FOR i FROM 1 TO 3 DO FOR j FROM 1 TO 3 DO MoveJ pOven{1, i, j}, v1000, fine, tool0; MoveJ pHome, v1000, fine, tool0; ENDFOR ENDFOR !conv2home FOR i FROM 1 TO 2 DO MoveJ pConv{1, i}, v1000, fine, tool0; MoveJ pHome, v1000, fine, tool0; ENDFOR !man2home MoveJ pMan{1}, v1000, fine, tool0; MoveJ pHome, v1000, fine, tool0; ENDPROC ENDMODULE |
| --- | --- |

A peripheral device used to control an industrial robot remotely. It is an HMI interface that can be used not only to configure and control the robot but also to program and design new capabilities and features. Figure 2 shows a virtual simulation of the ABB teach pendant.

Using GetTime(\Hour)*3600 + GetTime(\Min)*60 + GetTime(\Sec); to compute the time

To configure a programmable key as a digital variable it has to be set in Control Panel/ProgKeys option.